MICROCONTROLLER AND IOT LABORATORY

(GR24A3095) – RECORD OF PROGRAMS

Internet-Enabled (Firebase) Projects – Tasks 7 to 15

Department of Computer Science and Engineering

Gokaraju Rangaraju Institute of Engineering and Technology

# Experiment 7: Mobile App Simple Interface (Location Sensor + Text-to-Speech)

## Aim

To develop a mobile app simple interface using a Location Sensor and a Text-to-Speech converter, with an ESP32 publishing location data to Firebase.

## Components Required

ESP32, push-button, MIT App Inventor app (LocationSensor, TextToSpeech, FirebaseDB), Firebase Realtime Database.

## Program

/* Task 7: Mobile App simple interface  (a) Location Sensor  (b) Text-to-Speech

* This sketch is the ESP32/cloud half: it publishes a location (lat/long) to

* Firebase. A mobile app built in MIT App Inventor then reads this location and

* speaks it aloud using its TextToSpeech component (see README.md).

*

* A button nudges the demo coordinates so you can see live updates in Firebase.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define BTN 14

double lat = 17.4600;   // GRIET, Hyderabad (demo)

double lng = 78.3200;

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(BTN, INPUT_PULLUP);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

if (digitalRead(BTN) == LOW) { lat += 0.0005; lng += 0.0005; delay(200); }

String msg = "You are near GRIET, Hyderabad";

Serial.printf("Location: %.4f, %.4f\n", lat, lng);

String json = "{\"latitude\":" + String(lat, 4) + ",\"longitude\":" + String(lng, 4) +

",\"message\":\"" + msg + "\"}";

fbPatch("/mciot_lab/task07_location", json);

delay(2000);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task07_location

{

"latitude": 17.46,

"longitude": 78.32,

"message": "You are near GRIET"

}

## Wokwi Simulation

https://wokwi.com/projects/479121831366862849 (Task 1 ref)

# Experiment 8: Sensor/Activator Interfacing using ESP32 with MPU6050

## Aim

To interface the MPU6050 accelerometer and gyroscope with ESP32, drive an LED activator on motion, and upload the readings to Firebase.

## Components Required

ESP32, MPU6050 module, LED, 220 ohm resistor, Firebase Realtime Database.

## Program

/* Task 8: Sensor/Activator interfacing using ESP32 with MPU6050

* Reads accelerometer + gyroscope and uploads motion data to Firebase.

* An LED (activator) turns on when strong motion/tilt is detected.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <Wire.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define LED 2

Adafruit_MPU6050 mpu;

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(LED, OUTPUT);

if (!mpu.begin()) { Serial.println("MPU6050 not found!"); while (1) delay(10); }

mpu.setAccelerometerRange(MPU6050_RANGE_8_G);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

sensors_event_t a, g, temp;

mpu.getEvent(&a, &g, &temp);

float mag = sqrt(a.acceleration.x * a.acceleration.x +

a.acceleration.y * a.acceleration.y +

a.acceleration.z * a.acceleration.z);

bool motion = fabs(mag - 9.8) > 3.0;     // deviation from gravity => motion/tilt

digitalWrite(LED, motion ? HIGH : LOW);

Serial.printf("Accel(%.2f,%.2f,%.2f) Gyro(%.2f,%.2f,%.2f) Motion=%s\n",

a.acceleration.x, a.acceleration.y, a.acceleration.z,

g.gyro.x, g.gyro.y, g.gyro.z, motion ? "YES" : "NO");

String json = "{\"ax\":" + String(a.acceleration.x, 2) + ",\"ay\":" + String(a.acceleration.y, 2) +

",\"az\":" + String(a.acceleration.z, 2) + ",\"gx\":" + String(g.gyro.x, 2) +

",\"gy\":" + String(g.gyro.y, 2) + ",\"gz\":" + String(g.gyro.z, 2) +

",\"motion\":\"" + (motion ? "YES" : "NO") + "\"}";

fbPatch("/mciot_lab/task08_mpu6050", json);

delay(1500);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task08_mpu6050

{

"ax": 0.12,

"ay": -0.03,

"az": 9.79,

"gx": 0.01,

"gy": 0.0,

"gz": 0.02,

"motion": "NO"

}

## Wokwi Simulation

https://wokwi.com/projects/479123668226128897

# Experiment 9: Internet-enabled Remote Range Indicator

## Aim

To implement an internet-enabled remote range indicator using the HC-SR04 ultrasonic sensor with near/mid/far LED indication and Firebase upload.

## Components Required

ESP32, HC-SR04 ultrasonic sensor, 3 LEDs (green/yellow/red), 3 x 220 ohm resistors, Firebase Realtime Database.

## Program

/* Task 9: Internet-enabled Remote Range Indicator

* ESP32 + HC-SR04 ultrasonic sensor -> Firebase Realtime Database.

* Shows near/mid/far on 3 LEDs and uploads distance + status to the cloud.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

// ===== Firebase Realtime Database config =====

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define TRIG 5

#define ECHO 18

#define LED_GREEN 19

#define LED_YELLOW 21

#define LED_RED 22

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure();

HTTPClient h;

String url = "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH;

h.begin(c, url);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code);

h.end();

}

float readDistanceCm() {

digitalWrite(TRIG, LOW); delayMicroseconds(2);

digitalWrite(TRIG, HIGH); delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long dur = pulseIn(ECHO, HIGH, 30000);

return dur ? dur * 0.0343 / 2.0 : 400.0;

}

void setup() {

Serial.begin(115200);

pinMode(TRIG, OUTPUT); pinMode(ECHO, INPUT);

pinMode(LED_GREEN, OUTPUT); pinMode(LED_YELLOW, OUTPUT); pinMode(LED_RED, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected: " + WiFi.localIP().toString());

}

void loop() {

float d = readDistanceCm();

String status;

digitalWrite(LED_GREEN, LOW); digitalWrite(LED_YELLOW, LOW); digitalWrite(LED_RED, LOW);

if (d > 100)      { digitalWrite(LED_GREEN, HIGH);  status = "FAR"; }

else if (d > 40)  { digitalWrite(LED_YELLOW, HIGH); status = "MID"; }

else              { digitalWrite(LED_RED, HIGH);    status = "NEAR"; }

Serial.printf("Distance: %.1f cm | Status: %s\n", d, status.c_str());

String json = "{\"distance_cm\":" + String(d, 1) + ",\"status\":\"" + status + "\"}";

fbPatch("/mciot_lab/task09_range", json);

delay(2000);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task09_range

{

"distance_cm": 37.5,

"status": "NEAR"

}

## Wokwi Simulation

https://wokwi.com/projects/479124260821687297

# Experiment 10: Internet-enabled IR Sensor

## Aim

To implement an internet-enabled IR obstacle sensor that detects objects, counts detections and reports them to Firebase.

## Components Required

ESP32, IR obstacle sensor (push-button proxy in Wokwi), LED, 220 ohm resistor, Firebase Realtime Database.

## Program

/* Task 10: Internet-enabled IR Sensor

* ESP32 + IR obstacle sensor -> LED + Firebase Realtime Database.

* Wokwi has no IR obstacle part, so a push-button simulates the IR output

* (pressed = object detected = IR LOW). On real hardware use an IR module on the same pin.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define IR_PIN 14      // IR module OUT (active LOW when object detected)

#define LED 2

int count = 0;

bool last = false;

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(IR_PIN, INPUT_PULLUP); pinMode(LED, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

bool detected = (digitalRead(IR_PIN) == LOW);  // active low

digitalWrite(LED, detected);

if (detected && !last) count++;                // count rising detections

last = detected;

Serial.printf("Object: %s | Count: %d\n", detected ? "DETECTED" : "clear", count);

String json = "{\"object\":\"" + String(detected ? "DETECTED" : "CLEAR") +

"\",\"count\":" + String(count) + "}";

fbPatch("/mciot_lab/task10_ir", json);

delay(800);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task10_ir

{

"count": 3,

"object": "DETECTED"

}

# Experiment 11: Internet-enabled Smart Garden Maintenance

## Aim

To implement an internet-enabled smart garden system that reads soil moisture and temperature, auto-waters when dry, supports remote pump control from Firebase, and logs data to Firebase.

## Components Required

ESP32, soil-moisture sensor (potentiometer), DHT22, water pump/relay (LED), 220 ohm resistor, Firebase Realtime Database.

## Program

/* Task 11: Internet-enabled Smart Garden Maintenance

* ESP32 + soil-moisture (potentiometer) + DHT22 + water pump (LED/relay).

* Uploads readings to Firebase and auto-waters when soil is dry.

* Also reads /mciot_lab/task11_garden/pump_cmd from Firebase for remote control.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include "DHT.h"

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define SOIL_PIN 34     // potentiometer (analog soil moisture)

#define DHTPIN 15

#define PUMP 2          // LED = water pump / relay

#define DRY_THRESHOLD 40  // percent

DHT dht(DHTPIN, DHT22);

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

String fbGet(String path) {

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

int code = h.GET(); String p = (code == 200) ? h.getString() : ""; h.end(); return p;

}

void setup() {

Serial.begin(115200);

pinMode(PUMP, OUTPUT); dht.begin();

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

int raw = analogRead(SOIL_PIN);              // 0..4095

int moisture = map(raw, 0, 4095, 0, 100);    // percent

float t = dht.readTemperature();

// remote override from Firebase: "ON" forces pump on, "OFF" forces off, else auto

String cmd = fbGet("/mciot_lab/task11_garden/pump_cmd"); cmd.replace("\"", "");

bool pump;

if (cmd == "ON") pump = true;

else if (cmd == "OFF") pump = false;

else pump = (moisture < DRY_THRESHOLD);       // auto-water when dry

digitalWrite(PUMP, pump ? HIGH : LOW);

Serial.printf("Moisture=%d%%  Temp=%.1fC  Pump=%s (cmd=%s)\n",

moisture, t, pump ? "ON" : "OFF", cmd.c_str());

String json = "{\"moisture\":" + String(moisture) + ",\"temperature\":" + String(t, 1) +

",\"pump\":\"" + (pump ? "ON" : "OFF") + "\"}";

fbPatch("/mciot_lab/task11_garden", json);

delay(2000);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task11_garden

{

"moisture": 35,

"pump": "ON",

"pump_cmd": "AUTO",

"temperature": 29.4

}

## Wokwi Simulation

https://wokwi.com/projects/479125598104623105

# Experiment 12: Internet-enabled Display of Ambient Parameters

## Aim

To implement an internet-enabled system that measures ambient temperature and humidity using DHT22, displays them on an I2C LCD, and uploads them to Firebase.

## Components Required

ESP32, DHT22 sensor, 16x2 I2C LCD, Firebase Realtime Database.

## Program

/* Task 12: Internet-enabled Display of Ambient Parameters

* ESP32 + DHT22 (temp/humidity) -> I2C LCD + Firebase Realtime Database.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include "DHT.h"

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define DHTPIN 15

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

LiquidCrystal_I2C lcd(0x27, 16, 2);

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure();

HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code);

h.end();

}

void setup() {

Serial.begin(115200);

dht.begin();

lcd.init(); lcd.backlight();

lcd.setCursor(0, 0); lcd.print("Ambient Monitor");

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

delay(1500); lcd.clear();

}

void loop() {

float t = dht.readTemperature();

float hum = dht.readHumidity();

if (isnan(t) || isnan(hum)) { Serial.println("DHT read failed"); delay(2000); return; }

lcd.setCursor(0, 0); lcd.printf("Temp: %.1f C ", t);

lcd.setCursor(0, 1); lcd.printf("Humidity: %.0f%% ", hum);

Serial.printf("Temp=%.1fC  Humidity=%.0f%%\n", t, hum);

String json = "{\"temperature\":" + String(t, 1) + ",\"humidity\":" + String(hum, 0) + "}";

fbPatch("/mciot_lab/task12_ambient", json);

delay(2000);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task12_ambient

{

"humidity": 62,

"temperature": 30.1

}

# Experiment 13: Internet-enabled Home Safety and Security System

## Aim

To implement an internet-enabled home safety and security system using a PIR intruder sensor and a gas sensor that raises a buzzer alarm and reports the status to Firebase.

## Components Required

ESP32, PIR motion sensor, gas sensor (potentiometer proxy), buzzer, Firebase Realtime Database.

## Program

/* Task 13: Internet-enabled Home Safety and Security System

* ESP32 + PIR (intruder) + gas sensor (potentiometer proxy) + buzzer.

* Raises an alarm and reports status to Firebase Realtime Database.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define PIR 13

#define GAS_PIN 34        // potentiometer = gas level proxy

#define BUZZER 2

#define GAS_LIMIT 60       // percent

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(PIR, INPUT); pinMode(BUZZER, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

bool intruder = digitalRead(PIR);

int gas = map(analogRead(GAS_PIN), 0, 4095, 0, 100);

bool gasLeak = gas > GAS_LIMIT;

bool alarm = intruder || gasLeak;

digitalWrite(BUZZER, alarm ? HIGH : LOW);

Serial.printf("Intruder=%s  Gas=%d%%  Alarm=%s\n",

intruder ? "YES" : "NO", gas, alarm ? "ON" : "OFF");

String json = "{\"intruder\":\"" + String(intruder ? "YES" : "NO") +

"\",\"gas_level\":" + String(gas) +

",\"alarm\":\"" + String(alarm ? "ON" : "OFF") + "\"}";

fbPatch("/mciot_lab/task13_security", json);

delay(1500);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task13_security

{

"alarm": "ON",

"gas_level": 45,

"intruder": "YES"

}

## Wokwi Simulation

https://wokwi.com/projects/479125364510114817

# Experiment 14: Internet-enabled Ambient Light, Proximity and Gesture Sensor (APDS-9960)

## Aim

To implement an internet-enabled system using the APDS-9960 sensor to read ambient light, proximity and gestures and upload them to Firebase.

## Components Required

ESP32, APDS-9960 sensor (analog/button proxies in Wokwi), Firebase Realtime Database.

## Program

/* Task 14: Internet-enabled APDS-9960 (ambient light, proximity, gesture)

* ESP32 + APDS-9960 -> Firebase Realtime Database.

* Wokwi has no APDS-9960, so two potentiometers simulate ambient-light and

* proximity, and a button simulates a detected gesture. On real hardware,

* use the SparkFun_APDS9960 library on I2C (SDA=21, SCL=22) instead.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define LIGHT_PIN 34

#define PROX_PIN 35

#define GESTURE_BTN 14

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(GESTURE_BTN, INPUT_PULLUP);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

int light = map(analogRead(LIGHT_PIN), 0, 4095, 0, 1000);  // lux proxy

int prox = map(analogRead(PROX_PIN), 0, 4095, 0, 255);      // proximity 0..255

String gesture = (digitalRead(GESTURE_BTN) == LOW) ? "SWIPE" : "NONE";

Serial.printf("Light=%d lux  Proximity=%d  Gesture=%s\n", light, prox, gesture.c_str());

String json = "{\"ambient_light\":" + String(light) + ",\"proximity\":" + String(prox) +

",\"gesture\":\"" + gesture + "\"}";

fbPatch("/mciot_lab/task14_apds9960", json);

delay(1000);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task14_apds9960

{

"ambient_light": 540,

"gesture": "SWIPE",

"proximity": 120

}

# Experiment 15: Internet-enabled PIR Sensor

## Aim

To implement an internet-enabled PIR motion sensor that detects motion, indicates it on an LED and reports the status to Firebase.

## Components Required

ESP32, PIR motion sensor, LED, 220 ohm resistor, Firebase Realtime Database.

## Program

/* Task 15: Internet-enabled PIR Sensor

* ESP32 + PIR motion sensor -> LED indicator + Firebase Realtime Database.

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

const char* WIFI_SSID = "Wokwi-GUEST";

const char* WIFI_PASS = "";

String FIREBASE_HOST = "elearning-1408b-default-rtdb.asia-southeast1.firebasedatabase.app";

String FIREBASE_AUTH = "BV9noeROfWdPpnZ3oI6HAmHbb4kermXQqt8qYniG";

#define PIR 13

#define LED 2

void fbPatch(String path, String json) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure c; c.setInsecure(); HTTPClient h;

h.begin(c, "https://" + FIREBASE_HOST + path + ".json?auth=" + FIREBASE_AUTH);

h.addHeader("Content-Type", "application/json");

int code = h.sendRequest("PATCH", json);

Serial.printf("[Firebase] PATCH %s -> %d\n", path.c_str(), code); h.end();

}

void setup() {

Serial.begin(115200);

pinMode(PIR, INPUT); pinMode(LED, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); }

Serial.println("\nWiFi connected");

}

void loop() {

bool motion = digitalRead(PIR);

digitalWrite(LED, motion);

Serial.printf("Motion: %s\n", motion ? "DETECTED" : "none");

String json = "{\"motion\":\"" + String(motion ? "DETECTED" : "NONE") + "\"}";

fbPatch("/mciot_lab/task15_pir", json);

delay(1500);

}

## Firebase Output (Realtime Database)

Path: /mciot_lab/task15_pir

{

"motion": "DETECTED"

}

## Wokwi Simulation

https://wokwi.com/projects/479124956249024513